Dewatering conditions

These conditions are draft and are a tracked change version of the Applicant’s Proposed Conditions incorporating Panel amendments. Note that as separate consents are sought, the below represents an interpretation of the Panel’s Appendix A2 conditions with only conditions relevant to the Residential subdivision included. The structure of the Panel’s draft conditions has been adopted where applicable.

The following notes guide the reader as follows:

- Yellow highlights require information that is not currently available and will be finalised later in the process.

- Blue highlights refer to lot numbers that will change with the subdivision scheme revision.

- Greyed text relates to the retirement village (being the conditions the Panel would have imposed if it had found that consent could have been granted for this part of the Project).

- Bold and underlined text refers to insertions made by the Panel Draft Conditions

- Struckthrough text refers to deletions made by the Panel Draft Conditions

Resource Consent:		Insert Consent Reference

Grants to:				Matamata Development Limited

Commencement date:		Insert Decision Date

Lapse Date:			Five (5) years after commencement date

Expiry date:		This consent expires when construction activities cease on the Site for more than one (1) year.

Location:			Station Road, Matamata (Lot 1 Deposited Plan South Auckland 65481, Lot 2 Deposited Plan 567678, Lots 1 and 2 Deposited Plan 21055, Lots 4 and 5 Deposited Plan 384886, Lot 204 Deposited Plan 535395 and Lots 25 and 106 Deposited Plan 393306, Lot 3 Deposited Plan South Auckland 14362)

The activity: A water permit (pursuant to section 14 of the Resource Management Act 1991) to temporarily take groundwater for construction dewatering purposes and dust suppression in association with the development of approximately 95ha for residential purposes, a neighbourhood centre, a retirement village, two solar farms and ancillary infrastructure.

This consent should be read in conjunction with:

- Insert Consent Reference

- Insert Consent Reference

- Insert Consent Reference

- Insert Consent Reference

- Insert Consent Reference

- Insert Consent Reference

- Insert Consent Reference

- Insert Consent Reference

- Insert Consent Reference

- Insert Consent Reference

- Insert Consent Reference

And must be interpreted with guidance from the following glossary:

| ADP | Accidental Discovery Protocol |
| --- | --- |
| (The) Council | Waikato Regional Council |
| MP | Management Plan |
| DWMP | Dewatering Management Plan |
| RMA | Resource Management Act 1991 |
| Site | Collectively, Lot 1 Deposited Plan South Auckland 65481, Lot 2 Deposited Plan 567678, Lots 1 and 2 Deposited Plan 21055, Lots 4 and 5 Deposited Plan 384886, Lot 204 Deposited Plan 535395 and Lots 25 and 106 Deposited Plan 393306, Lot 3 Deposited Plan South Auckland 14362. |
| SQEP | Suitably Qualified and Experienced Person |
| WMP | Wastewater Management Plan |

And is subject to the following conditions:

## General Conditions

- The activities authorised shall be undertaken activity must be carried out in general accordance with the information and plans submitted by the Consent Holder in support of AUTHXXXXXXXX as listed in Table 1 and received by the Environmental Protection Authority, and subject to the in accordance with the application for resource consent, including any reports, plans, and further information (listed in Appendix [1]) provided by the Consent Holder, and in accordance with the following conditions of consent. In the event of a conflict between the documents in Table 1 and the conditions of these resource consents, the conditions of these resource consents shall prevail. Where there is any apparent conflict between the application and consent conditions, the consent conditions shall prevail.

- For the purposes of this consent, any reference to ‘Site’ means land legally described as Lot 1 Deposited Plan South Auckland 65481, Lot 2 Deposited Plan 567678, Lots 1 and 2 Deposited Plan 21055, Lots 4 and 5 Deposited Plan 384886, Lot 204 Deposited Plan 535395 and Lots 25 and 106 Deposited Plan 393306, Lot 3 Deposited Plan South Auckland 14362 prior to any further subdivision of the land.

- The Consent Holder must advise the Waikato Regional Council (the Council) in writing, at least five (5) working days prior to works commencing on Site, so that monitoring of the conditions of this consent can be undertaken.

- Advice note: All correspondence with the Council required by these conditions of consent should be sent via email to xxx@wrc.govt.nz with reference to consent number Insert Consent Reference.

The Consent Holder shall be responsible for all contracted operations relating to the exercise of this resource consent and shall ensure contractors are made aware of the conditions of this consent and their requirement to comply with those conditions.

A copy of this resource consent permit and any certified Management Plans (MP) shall must be kept on Site at all times that the works authorised by this resource consent permit are being undertaken and shall must be produced without unreasonable delay upon request from a servant or agent of WRC. the Council.

Any reference in these conditions to a New Zealand Standard includes any later New Zealand standard that amends or replaces it.

### Fees and Charges

The Consent Holder must pay to the consent authority Council any administrative charge fixed in accordance with Section 36 of the RMA, or any charge prescribed in accordance with regulations made under Section 360 of the RMA.

- Advice notes:

This includes the reasonable costs incurred by the Council arising from supervision and monitoring of this permit, e.g. routine inspection of the Site by Council officers or agents, liaison with the Consent Holder, responding to complaints or enquiries relating to the Site, and review and assessment of compliance with the conditions of consents.

That pursuant to Section 332 of the RMA, enforcement officers may at all reasonable times go onto the property that is the subject of this consent, for the purpose of carrying out inspections, surveys, investigations, tests, measurements or taking samples.

### Consent Lapse

Pursuant to clause 26(2) of Schedule 5 to the FTAA, the consents numbered RCXX shall lapse ten (10) years from the date of commencement unless it has been given effect to, surrendered, or been cancelled at an earlier date.

### Review

The WRC may during July each year from commencement of construction until 2 years after construction is completed The Council may, once per year, on any of the last five (5) working days of either May or November, serve notice on the Consent Holder under Section 128(1) of the RMA to review the conditions of this consent where::

- A material adverse effect which was not identified in the AEE (and supporting material for the resource consent application) has arisen; or

The magnitude of adverse effects from the project are materially larger than what was indicated in the AEE (and supporting material for the resource consent application).

Costs associated with any review of conditions of this consent will be recovered from the Consent Holder in accordance with the provisions of Section 36 of the RMA.

This consent does not attach to the land and is personal to Matamata Development Limited and Unity Management Limited. This consent must not be used by any other person or party without the express written approval of either Matamata Development Limited or Unity Management limited.

## Dewatering Parameters

Dewatering at each location (greenway, each WWPS and trenches) must only occur for the time required to complete the construction works (at each location).

The daily diversion volume authorised by this resource consent to construct the Greenway, wastewater pump stations, wet wells, and wastewater pipelines is 454 m3/day.

Ground water take volumes shall be in accordance with the Hydrogeological Assessment prepared by WGA titled ‘Ashbourne Development Hydrogeological Effects Assessment Appendix D and E dated June 2025.

The maximum dewatering depths and methodologies shall be in accordance with the Ashbourne Development Hydrogeological Effects Assessment dated June 2025 unless the WRC certifies otherwise.

The daily groundwater take for dust suppression purposes from the bore will be used for development of 10 to 15 ha at any one time at a rate of up to 336 m3/day for up to 168 days in a calendar year.

On all days when dewatering occurs, recharge trenches (if used) and/or settlement ponds shall be checked and maintained daily by the Consent Holder to achieve optimal infiltration capacity and groundwater recharge and/or surface water management at all times. The methods of monitoring and maintaining recharge trenches and/or settlement ponds shall be included in the certified GMCP or OMMP as relevant.

### Expert Supervision

The Consent Holder shall ensure that all dewatering and associated activities are supervised by suitably qualified and experienced practitioners and the site engineer.

### Waste Water Pump Stations

The wastewater pump station referred to as ‘WWPS Northern and WWPS Central’, inlet pipe, wet well and underground storage facility shall be designed in accordance with the layout and general accordance with the RITS requirements as set out in the Infrastructure Report prepared by Maven Associates titled ’XXXX’ , dated XX December 2025.

The dewatering for the excavation for the Central WWPS installs shall be to 60.83m RL and the maximum drawdown for the excavation for the Central WWPS install shall be to 61.43m RL.

The Consent Holder shall implement a water measuring system to quantify the volume of water taken on a continuous daily basis prior to works commencing that relate to the taking of water. The water measuring system shall have a reliable calibration to flow and be maintained to an accuracy of +/- 5%. Prior to commencing to take groundwater under this resource consent, evidence of the water measuring system’s calibration to an accuracy of +/- 5% shall be provided to the WRC.

Additional calibration of the water measuring system to ensure that the system has an accuracy of +/- 5% shall be undertaken by the Consent Holder at the written request of WRC.

Evidence documenting each respective additional calibration shall be forwarded to the WRC by the Consent Holder within one month of the calibration being completed.

During construction the Consent Holder shall maintain a continuous daily record of water taken for dewatering purposes. The record shall state:

- The groundwater take location;

The date on which the water was taken;

The total daily volume of water abstracted (m3);

Zero values when no water is being taken;

The total number of pumping hours per day; and

The maximum dewatering depth level.

Following construction, within one calendar month, the Consent Holder shall identify a minimum of 15 groundwater monitoring sites for long term monitoring of the groundwater level response. These will be focused on the existing monitoring bores. The selected bores will be chosen based on areas of observed initial response and recorded in the GMCP monitoring.

The Consent Holder shall monitor temporary dewatering monitoring sites during the pumped period and for a week following cessation of pumping.

Where no groundwater level response is observed the groundwater monitoring sites will be reviewed by a SQEP in hydrogeology three months after the construction phase ceases.  The site will be removed through a variation to the GMCP if monitoring of that site is no longer required.

An annual groundwater monitoring report will be prepared by the Consent Holder within one calendar month following completion of construction for a period of 2 years and provided to WRC.

That the TARP required in accordance with Condition [22] above shall set ground and surface water Alert and Alarm Trigger Levels for all dewatering activities. The details of which proposed groundwater level monitoring sites and methods and be set out as per the below:

Green – drawdown (below baseline) that is within predicted range

Amber – drawdown at the upper end of the predicted range

Red – drawdown exceeding the predicted range, and/or potentially triggering damaging settlement

The expected drawdown levels that inform the TARP shall consider the influence of background levels, including seasonal groundwater fluctuations at the time of monitoring.

In the event that a Green, Amber or Red alert level is triggered, the Consent Holder shall carry out the trigger level response actions as detailed in the certified GMCP, TARP and these conditions.

The Consent Holder shall submit all required baseline monitoring data to the WRC, as collected and recorded in accordance with the GMCP and associated TARP, prior to commencement of any dewatering activities.

## Management Plans

The following MP is required to be prepared/completed:

- Dewatering Management Plan (DWMP)

The Consent Holder must ensure that all MPs are prepared by a SQEP, where the MP is an update of an existing draft, the update must be completed by a SQEP.

The Consent Holder must submit the listed MPs to the Council for certification at least twenty (20) working days prior to work commencing.

MPs may be submitted in parts or in stages to address particular activities or to reflect the staged implementation of the development. When a MP is provided in part or for a stage it must satisfy all certification requirements, including submission to the Council for certification. MPs submitted to the Council must clearly show the linkage with MPs for adjacent stages and any interrelated activities or other MPs.

The certification process for the MPs must be confined to confirming that the MPs:

- Give effect to their objective/s (including any updated objective/s determined as part of a review provided for in Condition [(9)]).

Address the consent condition requirements.

Contain the required information.

Are generally consistent with the application documents (including draft MPs) listed in Appendix [1].

Within twenty (20) working days of receiving a MP for certification, the Council must:

- Notify the Consent Holder that the MP is certified.

Or, notify the Consent Holder that the MP is not certified, including the reasons why and the matters that must be addressed before this can occur. If further information is required, the Council will have a further ten (10) working days (from receipt of the further information) to confirm whether the MP has been certified.

If no response is received from the Council within twenty (20) working days of lodgement of any MP, the relevant MP will be deemed to be certified.

The Consent Holder must implement all certified MPs for the duration of the works.

### Amendments to Management Plans

Any changes and/or updates to a certified MP must be made in writing and submitted to the Council for certification in accordance with Condition [(36)].

While a MP is being changed/updated, a construction activity must cease unless the Council provides written confirmation that the activity may continue.

- Advice note: This condition does not relate to any operational aspect of a MP.

### Dewatering Management Plan (DWMP)

The Consent Holder will manage dewatering in accordance with a certified DWMP. The objective of the DWMP is to monitor the effects of dewatering on groundwater and to manage any unacceptable effects that may arise during dewatering. The DWMP will be updated as required to meet the objective. The DWMP must include, but is not limited to:

- A response to the conditions of this consent.

Maximum allowable dewatering pumping capacity and duration for each dewatering location.

Maximum allowable reduction in groundwater level allowed by dewatering at each dewatering location.

Any requirement for monitoring effects on groundwater levels adjacent to structures within 50 metres of the locations of proposed dewatering. This requirement must take into account the importance of the structure and its vulnerability to damage by settlement that could occur due to lowering of groundwater levels in its vicinity arising from dewatering.

Treatment requirements for the discharge of dewatered groundwater. At a minimum, this is to provide for removal of suspended sediment and may require monitoring pH with associated contingency measures should the pH fall outside an acceptable range.

Appropriate monitoring and reporting requirements.

The process for the ongoing review and amendment of the DWMP to maintain its effectiveness.

## Construction Conditions

### Pre- Start Requirements

The Consent Holder shall must appoint a representative(s) single Site Manager prior to commencement of any works authorised by this resource consent, who shall be WRC s ay works who must be the Council’s principal contact person in regard to matters relating to this consent. The Consent Holder shall must inform WRC the Council of the representative’s name and how they can be contacted prior to this consent being exercised. Should that person(s) change during the term of this resource consent, the Consent Holder shall must immediately inform the WRC, and shall also give written notice to the Council of the new representative’s name and how they can be contacted mobile phone number.

The following pre-start requirements must take place for each stage of development:

- At least ten (10) working days prior to commencement of construction on Site, the Consent Holder must provide to the Council:

An invitation to attend a pre-start meeting.

The name and contact details of the Site Manager and contractor.

The planned date, staging, and duration of construction.

The Consent Holder must, at least ten (10) working days prior to the commencement of construction, invite a representative(s) of Ngāti Hinerangi, Raukawa, and Ngāti Hauā to:

- Attend the pre-start meeting.

Provide a karakia prior to the commencement of Site works.

Undertake a cultural induction for key Site personnel.

Monitor earthworks. If the invitation to monitor earthworks is accepted, the Consent Holder must ensure that the monitoring office is provided with all bulk earthworks timetabling.

Prior to the commencement of activities authorised by this consent on the on Site, the Consent Holder shall must hold a pre-start meeting that:

- Is located on the subject Site.

- Is scheduled not less than ten (10) five working days (or such other timeframe that is agreed in writing between the Consent Holder and WRC) before the anticipated prior to the commencement of activities works.

- Includes:

- Representatives of the contractor/s who must undertake operations on Site

- All technical specialists who need to be present on Site during the works to manage/monitor works (e.g. engineer/s, ecologist etc.)

Outlines the strategy, sequence and approach to constructing each basin.

Confirms the approach to management plans and what is relevant per delivery stage(s).

Is attended by the site representative, the contractor, and any other relevant party representing the Consent Holder, including the Consent Holder’s Ecologist who will introduce and explain the relevant ecological management plans, implementation and purpose.

At least ten (10) working days prior to the meeting, shall invite the following parties to the pre-start meeting:

  - WRC compliance monitoring officer[s] and engineers; and

ii.	representatives from Ngāti Hauā, Ngāti Hinerangi, and Raukawa.

### Accidental Discovery Protocol (ADP)

In the event that any archaeological Sites, remains, artefacts, taonga (Maaori artefacts) or kōiwi are unearthed, dislodged, uncovered or otherwise found or discovered during the earthworks (‘the discovery’), the Consent Holder must implement an ADP which must consist of the following actions:

- Cease works immediately in all parts of the Site affected by the discovery.

Advise Ngāti Hinerangi, Raukawa, Ngāti Hauā, and Waikato-Tainui and Council within one (1) day of the discovery.

Arrange for a SQEP archaeologist to attend Site to confirm if the material is archaeological in nature or involves kōiwi.

Contact the NZ Police, Coroner and HNZPH as appropriate.

Undertake specific preservation measures to address any discovery that includes water-logged or wet archaeological materials.

Not recommence works in the parts of the project Site affected by the discovery until all necessary statutory authorisations or consents have been obtained.

### Complaints

That if any complaints are received by the Consent Holder regarding the works authorised by this consent, the Consent Holder must record the following details in a Complaints Log:

- Date, time and type of complaint, including details of the incident, e.g. duration, any effects noted.

Name, address and contact phone number of the complainant (if provided).

Location from which the complaint arose.

The weather conditions and wind direction at the time of any dust or noise complaint.

The likely cause of the complaint.

The response of the Consent Holder including any corrective action undertaken by the Consent Holder.

Future actions proposed as a result of the complaint so as to avoid reoccurrence.

The Consent Holder must notify the Council of any complaint received that relates to the activities authorised by this resource consent as soon as reasonably practicable and no longer than two (2) working days after receiving the complaint.

The Consent Holder must respond to any complainant as soon as is reasonably practicable and, within five (5) working days, advise the Council and the complainant of the outcome of the Consent Holder’s investigation and all measures taken, or proposed to be taken, to respond to the complaint.

## Monitoring and Reporting

The Consent Holder must ensure that all dewatering and associated activities are supervised by suitably qualified and experienced practitioners and the site engineer.

The Consent Holder must install monitoring groundwater sites in accordance with the certified DWMP.

During construction, the Consent Holder must maintain a continuous daily record of dewatering and dust suppression activities. The record must state:

- The groundwater take location.

The date on which the water was taken.

The total daily volume of water abstracted (m3).

Zero values when no water is being taken.

The total number of pumping hours per day.

The maximum dewatering depth level.

The Consent Holder shall provide the results of the GMCP groundwater and the respective TARP’s monitoring to the WRC on a by monthly basis for the duration of works associated with this consent. The results shall be presented in a format which shows the monitoring results compared against trigger levels, along with a summary analysis of any data trends and forecasts.

## Advice Notes

  - The construction, operation, maintenance and management of the Ashbourne Greenway, pump stations and is authorised by a suite of resource and land use consents that operate in an integrated manner and must be read and implemented together. Reference should be made to all consents in the suite. These are:

    - AUTHXXXXXXXX

    - AUTHXXXXXXXX

    - …etc

  - This resource consent does not give any right of access over private or public property. Arrangements for access must be made between the Consent Holder and the property owner.

  - This resource consent is transferable to another owner or occupier of the land concerned, upon application, on the same conditions and for the same use as originally granted (Sections 134 to 137 of the RMA).

  - The reasonable costs incurred by WRC arising from supervision and monitoring of this/these consents will be charged to the Consent Holder. This may include but not be limited to routine inspection of the site by WRC officers or agents, liaison with the Consent Holder, responding to complaints or enquiries relating to the site, and review and assessment of compliance with the conditions of consents.

  - Note that pursuant to Section 332 of the RMA, enforcement officers may at all reasonable times go onto the property that is the subject of this consent, for the purpose of carrying out inspections, surveys, investigations, tests, measurements or taking samples.

All Management Plans lodged with the application are DRAFT and subject to change through the certification process.