## PROPOSED CONDITIONS OF CONSENT (OFF-SHORE APPLICATION)

Updated Draft:	7 July 2023

## CONDITIONS

### Consent Glossary

| Extraction Exclusion Area | The area where sand extraction is not permitted until allowed under condition 14 as shown on the Bioresearches Drawing “Proposed Offshore Sand Extraction Area showing Monitoring and Extraction Cells, and Control Areas with Monitoring Cells”, dated 6/7/2023. Appendix Four |
| --- | --- |
| ASEA | Approved Sand Extraction sub-Area. |
| Control Areas | The control areas defined on the Bioresearches Drawing “Proposed Offshore Sand Extraction Area showing Monitoring and Extraction Cells, and Control Areas with Monitoring Cells”, dated 6/7/2023. Appendix Four |
| Council | Auckland Council |
| CLG | Community Liaison Group |
| EMMP | Environmental Monitoring Management Plan |
| Extraction Area | The consented sand extraction area as defined by the following coordinates: |

Point	New Zealand Transverse Mercator Projection	World	Geodetic	System	1984 (G1762)

| Point ID | Northing (NZTM) | Easting (NZTM) | LAT (Y) | LONG (X) |
| --- | --- | --- | --- | --- |
| 1 | 5994435.59 | 1755496.67 | 36 10 56.58667 S | 174 43 45.14230 E |
| 2 | 5992572.41 | 1753257.05 | 36 11 58.31749 S | 174 42 16.82793 E |
| 3 | 5994126.25 | 1751721.20 | 36 11 08.81017 S | 174 41 14.26562 E |
| 4 | 5998824.36 | 1748945.94 | 36 08 37.94140 S | 174 39 19.99142 E |
| 5 | 6000863.22 | 1747812.50 | 36 07 32.41856 S | 174 38 33.27521 E |
| 6 | 6001992.28 | 1747352.70 | 36 06 56.01146 S | 174 38 14.13137 E |
| 7 | 6003277.52 | 1748977.20 | 36 06 13.42132 S | 174 39 18.21223 E |

| MBES | Multi Beam Echo Sounder |
| --- | --- |
| MMMP | Marine Mammal Management Plan |
| Monitoring Cell | Subdivisions of the Extraction Area as defined on the Bioresearches Drawing “Proposed Offshore Sand Extraction Area showing Monitoring and Extraction Cells, and Control Areas with Monitoring Cells”, dated 6/7/2023 or any subsequent revisions. Appendix Four |
| PSEA | Proposed Sand Extraction sub-Area |
| PSEAR | Pre-Sand Extraction Assessment Report |
| Reporting Cell | Subdivisions of the Extraction Area as defined on the Bioresearches Drawing “Proposed Offshore Sand Extraction Area showing Monitoring and Extraction Cells, and Control Areas with Monitoring Cells”, dated 6/7/2023. Appendix Four |
| RMA | Resource Management Act 1991 |
| SEMR | Sand Extraction Monitoring Report |
| Sensitive Benthic Communities | Means the habitats described in the Appendix Two to these conditions. |
| SQEP | Suitably Qualified and Experienced Person |

# General Conditions

These conditions apply to all resource consents.

- These consents must be carried out in general accordance with the documents and drawings and all supporting additional information submitted with the application and as updated during the Council Hearing and appeal process relating to applications referenced by the Council as resource consent application numbers CST60343373 and DIS60371583 (consent numbers to be updated):

  - The documentation included in Appendix One (to be updated)

  - Where there is an inconsistency between documentation then the later documentation will apply.

- Where there are differences or apparent conflicts between the documents listed in Condition 1 and the conditions below, the conditions below will prevail.

- Under section 125 of the RMA, these consents lapse 24 month from the date of these consents unless:

  - The consents are given effect to; or

  - The Council extends the period after which the consents lapse.

- The Consent Holder must pay the Council an initial consent compliance monitoring charge of $(TBC)(inclusive of GST), plus any further monitoring charge or charges to recover the actual and reasonable costs incurred to ensure compliance with the conditions attached to these consents.

#### Advice Note

The initial monitoring deposit is to cover the cost of inspecting the site, carrying out tests, reviewing conditions, updating files, etc., all being work to ensure compliance with the resource consent(s). In order to recover actual and reasonable costs, monitoring of conditions, in excess of those covered by the deposit, will be charged at the relevant hourly rate applicable at the time. The consent holder will be advised of the further monitoring charge. Only after all conditions of the resource consent(s) have been met, will the council issue a letter confirming compliance on request of the consent holder.

### Duration of the consent

- These consents will expire 20 years from the date on which notice is given in accordance with Condition 6 unless they have lapsed, been surrendered or been cancelled at an earlier date pursuant to the RMA.

### Commencement of the consents and surrender of existing consents

- The Consent Holder must notify the Council in writing at least ten (10) working days prior to sand extraction commencing under this consent.

- The Consent Holder must notify the proposed members of the CLG (as listed in Condition 66 in writing at least ten (10) working days prior to sand extraction commencing under this consent.

### Provide for a review under section 128

- The conditions of this consent may be reviewed by the Council (Manager - Resource Consents) at the Consent Holder’s cost, by serving notice under section 128(1) of the RMA within three months following receipt by the Council of an SEMR:

  - To deal with any significant unanticipated adverse effects on the environment arising from the exercise of these consents, or

  - To deal with any significant unanticipated adverse effect on the environment which is identified in any SEMR provided to the Council in the previous three months; and

- To require any remedial actions or alterations to the extraction and discharge activities to rectify that significant unanticipated adverse effect including, but not limited to, a reduction in the consented area.

### Occupancy of the common marine and coastal area

- The occupation of the common marine and coastal area by the authorised activities is not an exclusive right of occupancy. The general public or any person(s) may not be excluded from the area(s) or any part of the area(s) to which this consent applies.

### Extraction Exclusion Area

- A bathymetric survey of the Extraction Exclusion Area is to be undertaken during October annually using a multi-beam echosounder (MBES) capable of detecting seabed features to 0.5m in depth, with horizontal positional accuracy of 3m or better. The MBES survey output is to comprise detailed cross-sections across the swales, including relative seabed levels and chainages, water depths, cross-section start and end coordinates and comparing the series of annual surveys. It is also to comprise a detailed colour-banded bathymetric map of trenches and the 100m strip on each side, with a coordinate grid, showing the trenches and cross-section locations. The vertical resolution of the colour banding should be 0.2m. MBES backscatter data is to be included in the map (or a separate map) as well as bathymetry if the bathymetric data alone is not sufficient to identify trench features and compare them with previous annual maps.

- The outputs of the annual survey required under Condition 10 are to be submitted to Council within three months of the completion of the survey.

- The requirements of Condition 10 do not apply to any part of the Extraction Exclusion Area where a survey or a PSEAR has shown that the trenches have recovered to within -0.5m (vertical measure) of the surrounding seabed level provided that an assessment by a suitability qualified and experienced person confirming the recovery has been submitted to Council.

- The requirements of Condition 10 do not apply to any part of the Extraction Exclusion Area which has been subject to a PSEAR during the preceding 12 months.

- The Consent Holder must not undertake any further sand extraction from any part of the Extraction Exclusion Area until a survey confirms that the recovery standards set out in Condition 12 have been met, and the survey has been submitted to the Council in accordance with Condition 12.

# Operational Conditions

### Extraction area

- Sand extraction must be limited to ASEAs within the Extraction Area.

- Within the ASEA’s sand extraction must not occur in areas which contain any of the following:

  - Sediment with an average proportion of mud (grain size finer than 0.063 mm) exceeding 20% by weight; or

  - Sensitive benthic communities; or

  - The presence of benthic species absolutely protected under the provisions of the Wildlife Act 1953.

### Sand extraction volume

- The total volume of sand extracted (which is the sand which is loaded into the extraction vessel and transported from the site) during the life of the consent must not exceed 2,000,000m3. This volume limit will be reduced by any volume extracted under consents (consent numbers to be inserted) (Temporary Offshore consent). Sand extraction between the landward boundary of the Extraction Area and the 30m isobath (from MSL) must not exceed 75,000m3 of sand during any 12-month period.

- In the event that sand extraction within a single Reporting Cell in an ASEA reaches 20,000m3 in any 12-month period then no further sand extraction from that Cell is permitted for the following 12 months.

### Sand extraction and discharge methodology

- The sand extraction and transportation is to be carried out by the William Fraser using a trailer suction dredge unless a change in the vessel(s) used for sand extraction and transportation has been certified in writing by the Council in accordance with Condition 21.

- The Consent Holder must, to their best endeavours, evenly spread extraction within the Approved Sand Extraction Areas.

- Any change of the sand extraction and/or discharge methodology from that provided in the consent application documentation and/or the use of an alternative vessel(s) for extraction to the William Fraser will require written certification from the Council, before any change in the sand extraction operation or vessel(s) used for the sand extraction. Council shall have 20 working days from receiving the request for the change to certify or otherwise respond to the proposed changes to the sand extraction and/or discharge methodology or vessel. If no response has been provided by Council within 20 working days from receiving the request for the change, the proposed changes shall be deemed approved by Council.

- Any proposed change of vessel or extraction and/or discharge methodology must be notified in writing to the Council and be accompanied by a report prepared by a suitably qualified and experienced person that demonstrates any proposed change of the sand extraction or discharge method and/or a change to a different extraction vessel will not result in any materially different adverse effects from the vessel and methodology then in use and must include a report from an independent engineering surveyor stating the volume of sand in cubic metres that is carried by the vessel(s) for extraction and transport when loaded to the load line marked on the vessel’s or vessels’ hull(s) in accordance with Maritime NZ requirements.

#### Advice Note

An alternative vessel is only required to be certified once unless there are subsequent significant changes to the sand extraction and/or discharge methodology used on that vessel.

### Operational time restriction

- Sand extraction must only occur between the hours of:

  - 9:00pm to 5:00am during the period when New Zealand daylight saving is in force; or

  - 7:00pm and 6:00am during the period when New Zealand daylight saving is not in force.

Despite the restriction in Condition 23 there will be no operational time restriction for up to 20 days per calendar year (excluding statutory public holidays) (exception to daily time limits). If any of those 20 days are used consecutively, the consecutive period shall not exceed 5 days. At the conclusion of any consecutive period of days used, an equivalent period of consecutive days will accrue before the exception to daily time limits is again relied upon.

# Management Plans

### Environmental monitoring management plan

- Prior to the preparation of the first PSEAR, the Consent Holder must submit to the Council an EMMP for certification that the monitoring to be undertaken in accordance with the EMMP will achieve the objectives of the EMMP and compliance with the relevant consent conditions.

- Any subsequent review or updates of the monitoring methodologies proposed in the EMMP must be submitted to the Council for certification prior to being implemented. Any other updates to the EMMP (including final PSEAR and SEMR reports) must also be submitted to the Council so that Council can maintain a current copy of the EMMP.

- The Consent Holder shall advise Council that an EMMP is being submitted at least 10 working days prior to the EMMP being submitted. Council shall then have 20 working days from the information being submitted to certify or otherwise respond to the EMMP and any subsequent modifications. If no response has been provided by Council in 20 working days from receiving the EMMP and any subsequent modifications, the EMMP shall be deemed certified by Council.

- The Consent Holder must have the EMMP(s) peer reviewed by a SQEP(s) prior to its submission to the Council. The Consent Holder must provide any feedback received from the SQEP(s) on the EMMP to the Council at the time they are submitted for certification, along with any explanation of where suggested changes to the EMMP by the SQEP(s) has not been incorporated and the reasons why.

- The Consent Holder must meet the costs of the production, certification and subsequent updating of the EMMP. The EMMP will be based on the draft EMMP (date to be inserted). Any changes to the final EMMP submitted for certification to Council

which differ from the draft EMMP (date to be inserted) will be supported by an assessment from an appropriately qualified and experienced person or persons.

- The objectives of the EMMP are:

  - To require a PSEA

    - To identify those sub-areas within a Proposed Sand Extraction Area suitable for sand extraction and those areas which are to be excluded in accordance with Condition 16.

    - To provide the baseline information for the subsequent sand extraction monitoring.

  - To require sand extraction monitoring

    - To identify in a timely manner any changes required to the sand extraction method and timing to further minimise any identified significant unanticipated adverse ecological and/or coastal processes effects on the environment.

    - To assess the continued suitability of any currently approved sand extraction sub-areas.

    - To assess if any changes are required to the monitoring and reporting methodologies.

  - To require sand extraction and vessel tracking monitoring

    - To provide a record of sand extraction volumes, locations (i.e. reporting cells), timing, water depth and sea conditions during extraction and confirmation that the permitted sand extraction volumes are being complied with.

    - To identify when sand extraction monitoring is required to be undertaken.

    - To provide a record of where sand extraction has been undertaken and confirmation that sand extraction has only been undertaken within ASEAs.

    - To demonstrate that best endeavours have been used to evenly spread extraction throughout the ASEAs.

- The EMMP must:

  - Record the objectives of the EMMP and the monitoring programme to achieve those objectives.

  - Include a plan showing the Extraction Area ASEAs, Reporting Cells and Monitoring Cells.

  - Include a table defining the maximum quantity of sand to be extracted and the volume which has been extracted from each ASEA.

  - Detail the pre-sand extraction monitoring programme for the PSEAR required under Condition 38 for each PSEA. The monitoring programme must be sufficient to achieve the objectives set out in Condition 29(a), and must include, at minimum, details on the following requirements:

    - That all pre-extraction baseline ecological and subsequent monitoring surveys are undertaken using a consistent quantitative sampling technique;

    - Control and potential impact sites for baseline data collection;

    - For the purpose of analysis and reporting, taxa must be identified to the lowest practical taxonomic levels in order to obtain ecologically meaningful results;

    - The proposed reporting regime for the results of the monitoring, which, as a minimum, must include a final reporting date six months from the completion of the monitoring programme, and may include interim reporting dates;

    - Bathymetric survey method for the sea floor (pre-sand extraction) in the monitoring cells in which sand extraction will occur and the immediately surrounding monitoring cells (which is to include seabed imaging in map form using 0.2m vertical resolution MBES colour-banded bathymetry). MBES backscatter is to be used, in addition to MBES bathymetric data, to allow identification, and comparison over time, of seabed features; and

    - Sediment texture monitoring.

  - Detail the monitoring programme for the SEMR required under Condition 44. The monitoring programme must be sufficient to achieve the objectives set out in Condition 29(b), and must include, at minimum:

    - Bathymetric survey method for the sea floor (post-sand extraction) in the monitoring cells in which sand extraction will occur and the immediately surrounding monitoring cells (which is to include seabed imaging in map form using 0.2m vertical resolution MBES colour-banded bathymetry). MBES backscatter is to be used, in addition to MBES bathymetric data to allow identification, and comparison over time, of seabed features;

    - Sediment texture monitoring methodology;

    - The monitoring methodology for the collection of information capable of detecting whether the sand extraction is having effects of ecological significance upon benthic macrofauna and/or benthic communities; and

    - Set out the foredune and beach monitoring programme as required under Conditions 48 - 50 including the methodology, locations, timing and reporting requirements of beach topographic surveys.

  - Include copies of any completed PSEAR and SEMR.

  - Identify a process for assessing any issues arising from the survey design, methods of sampling, analysis and/or reporting and the process for implementing any recommended changes.

  - Specify the qualifications and experience required for those supervising and undertaking monitoring and reporting.

- The EMMP required by Conditions 24 to 28 must be reviewed at least every five years by the Consent Holder. The reviewed EMMP is to be submitted to the Council for certification in accordance with Conditions 25 to 28.

#### Advice Note

The EMMP is a “living document” that includes all monitoring documentation.

### Marine Mammal Management Plan

- At least five working days prior to sand extraction commencing under Condition 6, the Consent Holder must submit to the Council for certification a final Marine Mammal Management Plan (MMMP) to minimise the risk of harm to marine mammals from its operations. The MMMP must be based on the draft MMMP dated 08/12/2020. for certification that the MMMP is in accordance with Condition 32. Council shall have 20 working days from receiving the final MMMP to certify or otherwise respond to the MMMP. If no response has been provided by Council in 20 working days from receiving the MMMP, the MMMP shall be deemed certified by Council.

- The MMMP must detail the following:

  - Methods employed to minimise the risk of whale strike;

  - Methods employed to avoid the attraction of marine mammals to the extraction vessel;

  - Methods employed to minimise entanglement of marine mammals with the dredgehead and associated underwater equipment.

  - Methods to address any effects of the activity of sand extraction on the values associated with tohorā (whales) as a taonga species within the Embayment.

Any subsequent review or updates of the MMMP must be submitted to the Council for certification. Council shall have 20 working days from receiving the information to certify or otherwise respond to the updated MMMP. If no response has been provided by Council in 20 working days from receiving the information, the MMMP shall be deemed certified by Council.

- The Consent Holder must record and report any incident which results in injury or mortality to a marine mammal to the Council, Ngāti Manuhiri, Te Uri o Hau and the Department of Conservation as soon as practicable.

#### Advice Note

This consent does not remove the need to comply with the Wildlife Act 1953 and the Marine Mammal Protection Regulations 1992.

## MONITORING AND REPORTING

### Volume and location

- Prior to exercising this consent, the Consent Holder must provide to the Council a report prepared by an independent engineering surveyor stating the volume of sand in cubic metres that is carried by the William Fraser when loaded to the load line marked on the vessel’s hull in accordance with Maritime NZ requirements.

- The Consent Holder must keep a record for each extraction event of:

  - In the event that the William Fraser is not fully loaded, the Consent Holder may report the volume of the incomplete load calculated from the onboard sensors (Holland Dredge Design Sensors) measuring compliance with the load line marked on the vessel’s hull in accordance with Maritime NZ requirements before any unloading of sand and in any event within 30 minutes of the William Fraser tying up at the Port of Auckland. The record must include for each load:

    - A screenshot or other verifiable way of showing the date and time, the reading of the sensors and the draft of the vessel,

    - The volume of the load by reference to the load-line.

  - Subject to Condition 35, the volume of sand loaded into the extraction vessel shall be recorded on the basis that the maximum capacity of the hopper on the William Fraser is on average 900m3 of sand per extraction event, or such greater or lesser amount as is determined by the engineering surveyor’s report required by Condition 35.

  - The volume of sand from each Reporting Cell where extraction has occurred,

  - The date, time, water depth and sea conditions during the period of extraction,

  - An electronic record of the track of the sand extraction vessel (using a GPX file format or equivalent) and mapped using a differential global positioning system (“DGPS”) showing:

    - A complete track of the vessel from the southernmost waypoint shown in Appendix 4 prescribing the shoreward limits of its approach to and departure from the ASEAs.

    - A track of the vessel showing when the dredgehead is on the seabed extracting sand and when the dredgehead is above the seabed and not extracting sand for the purpose of turning outside the boundaries of the Extraction Area, avoiding the Extraction Exclusion Area, avoiding Sensitive Benthic Communities or protected species (see Condition 16) or for any other reason.

- The Consent Holder must provide to the Council a copy of the records required by Condition 36 quarterly from and including the first full quarter after these consents commence. If no sand extraction has occurred during that quarterly period then a statement to that effect will be provided to the Council. The Consent Holder must notify Council of any non-compliances with Conditions 16, 18, 19 and 23 and the reason for the non-compliance as soon as practical.

### Pre-sand extraction assessment report (PSEAR)

- Prior to extracting sand from any PSEA the Consent Holder must complete a PSEAR for that PSEA in accordance with the certified EMMP. The purpose of the PSEAR is:

  - To identify within a PSEA any areas of the seafloor which are unsuitable for sand extraction in accordance with Condition 16.

  - If the PSEAR overlays any part of the Extraction Exclusion Area which has not been confirmed to have recovered in accordance with Conditions 11 to 13, identify any trenches which have not recovered to within -0.5m (vertical measure) of the surrounding seabed level.

  - To provide baseline information for analysis against subsequent post-dredging monitoring (geomorphological monitoring, sediment texture and benthic monitoring) in sufficient detail for independent statistical or other expert assessment; and

  - To identify areas which are to be excluded from extraction for the reasons listed in Condition 16.

#### Advice Note

Any survey of the Extraction Exclusion Area forming part of a PSEAR is to be undertaken in accordance with the methodology outlined in Condition 10.

- The PSEAR must include but not be limited to:

  - Geomorphology

    - Identify within the PSEA, either by reference to established data or by reference to seabed sampling or surveys taken:

      - Any inferred pathways for sediment transport;

      - Areas of ripples on the seafloor;

      - Areas of the seafloor where the average proportion of mud (grain size finer than 0.063 mm) in samples exceeds 20% by weight; and

      - Where relevant, those areas within the Extraction Exclusion Area to be excluded from sand extraction.

  - Benthic habitat monitoring

    - Identify from information collected throughout the PSEA any areas where Sensitive Benthic Communities and /or benthic macrofauna, of particular conservation significance (for example, stony corals) or ecological sensitivity (for example, shellfish beds) exist and the presence of benthic species absolutely protected under the Wildlife Act 1953, and if so the degree of their ecological significance and the extent of their presence.

  - Map

    - Include a map showing those areas within the PSEA that are ASEAs and a map showing the areas of identified Sensitive Benthic Communities and benthic macrofauna.

    - Maps of the bathymetry and backscatter imagery of the PSEA.

- The Consent Holder must have the PSEAR peer reviewed by a SQEP(s) prior to its submission to the Council. The Consent Holder must provide any feedback received from the SQEP(s) on the PSEAR documents to the Council at the time they are

submitted for certification, along with any explanation of where any comment suggesting changes to the PSEAR has not been incorporated and the reasons why.

- The completed PSEAR is to be submitted to the Council for certification that the PSEAR meets the purpose of the PSEAR stated in Condition 38 and requirements under Condition 39 prior to sand extraction occurring within any part of the PSEA which has been identified as the ASEA.

### Initial pre-sand extraction reporting requirements

- The initial PSEAR must be prepared within the 12 months prior to commencement of sand extraction.

### Ongoing pre-sand extraction reporting requirements

- A PSEAR is to be completed for any PSEA where sand extraction has occurred previously but has not been undertaken within that PSEA for a period of greater than 12 months.

### Sand extraction monitoring report (SEMR)

- The Consent Holder must prepare and submit to the Council a SEMR for those ASEA’s where sand extraction has occurred since the completion of the previous SEMR on an annual basis for the first three years of these consents, followed by reports on years 5, 7 and 10 and then at five yearly intervals thereafter.

- The Consent Holder must have the SEMR(s) peer reviewed by a SQEP(s) prior to its submission to Council. The Consent Holder must provide any feedback received from the SQEP(s) on the SEMR documents to Council at any time they are submitted for certification, along with any explanation of where any comment suggesting changes to the SEMR has not been incorporated and the reasons why.

- The SEMR must be submitted to the Council within six months of each of the sand extraction timeframes listed in Condition 44 being reached.

- The SEMR must comply with the relevant requirements set out in the EMMP, and include:

  - An analysis of the results of the monitoring required under the approved EMMP and an assessment to ascertain whether extraction activity has adversely affected sediment transport processes and/or impacted on benthic macrofauna beyond impacts experienced as a result of natural perturbations;

  - A comparative analysis of the bathymetry within the limits of the survey accuracy;

  - A comparative analysis of sediment texture at sites within and adjacent to areas where sand extraction has been undertaken;

  - Any recommendations for changes to sand extraction or discharge methodologies (including rates and periods between sand extraction episodes) to avoid adverse effects on communities and species described in Condition 16 based on the results of the SEMR; and

  - The results of the foredune and beach monitoring.

### Foredune and Beach Monitoring

- The Consent Holder must undertake a topographical survey of the beach and foredune every six months with one survey to be undertaken in March-April period and one in September-October period to enable consistent seasonal comparisons. The topographical survey to include the following:

  - Extend alongshore from approximately 2.5 km north of the existing profile P1 to approximately 2.0 km south of existing Profile P8, as shown in Drawing IZ111900-003 Survey Beach Volume Compartments (Appendix Three).

  - The topographic survey is to be undertaken by a survey specified aerial drone utilising Real Time Kinetic (RTK) survey accuracy to 50mm horizontal and vertical and using secondary ground control points for validation. Survey point density to be no less than 1m between points across the whole survey area. The survey area shall include the:

    - foredune line (i.e. top of the foredune face);

    - seaward dune toe (i.e. major change of slope at dune/beach boundary);

    - high tide or most recent storm run up limit (i.e. berm location);

    - lowest position possible on the beach (i.e. low tide run-up limit); and

    - locations of Pakiri River, Poutawa Stream, and Te Arai Creek mouth channels through the foredune and across the beach.

- Analysis of the topographical surveys:

  - Analysis of the topographical survey data must be undertaken by dividing the beach/dune environment into the following three compartments as shown on Drawing IZ111900-003 (Appendix Three):

    - Northern compartment from 2.5km north of existing Profile P1 to the northern side of Te Arai Point.

    - Central compartment from the southern side of Te Arai Point to the northern side of Poutawa Stream.

    - Southern compartment from the south saide of Poutawa Stream to a point 2km south of existing Profile P8.

  - The analysis of the topographical survey data must include:

    - Interpolated Beach profiles at the historical beach profiles P1 to P9 as shown on Drawing IZ111900-003 (Appendix Three).

    - Additional Interpolated beach profile sites as determined in the EMMP.

    - Beach elevation and volume changes in designed areas around the mouths of Pakiri River, Poutawa Stream, and Te Arai Creek as shown in Drawing IZ111900-003 (Appendix Three).

    - Beach and foredune contour position and volume calculations over the topographic survey area within each of the above compartments.

- The Consent Holder will submit the results of the topographical survey to the Council no later than 2 months following the periods referred to in Condition 48.

## OTHER OPERATIONAL REQUIREMENTS

### Noise

- The noise (rating) level and maximum noise level generated by the sand extraction vessel (including any pumping or mechanical equipment used in the sand extraction process) must not exceed the following noise limits when measured and assessed on land at the adjacent coastline and/or within any notional boundary of a site:

7am-10pm (Monday to Sunday)	50dB LAeq

10pm-7am (Monday to Sunday)	40dB LAeq and 75dB LAmax

All noise measurements and assessments must be in accordance with the New Zealand Standard NZS 6801:2008 Measurement of environmental sound and the New Zealand Standard NZS 6802:2008 Acoustics – Environmental noise’. The notional Boundary is defined in Chapter J of the Auckland Unitary Plan.

### Lighting

- For all vessels associated with the sand extraction, to avoid or mitigate adverse effects on sea birds and people viewing from land, lighting is to be inward and downward facing and minimised as far as practicable while still complying with any relevant maritime regulations and safety requirements.

### Oil Spill Response Plan

- At least 20 working days prior to sand extraction commencing under Condition 6, the Consent Holder must submit to the Council Tier 1 (under the Maritime Transport Act 1994) Oil Spill Response Management Plan. This plan must be prepared in accordance with the requirements of Maritime New Zealand.

### Operational Schedule

- The Consent Holder must make a provisional copy of each weekly/monthly operating schedule available online (on the Consent Holder’s website) for public

viewing. The schedule must also provide details of monitoring, surveying and other non-sand extraction activities authorised under these consents.

#### Advice Note

Weekly operational schedules are subject to change as a result of weather, operational issues and other unforeseen events. For clarity, publishing the operating schedule on the Consent Holder’s website is considered sufficient to meet this condition of consent.

### Compliance

- The consent Holder shall comply with the requirements of all management plans prepared and certified by the Council pursuant to the conditions of these consents.

### Biosecurity

- Before the commencement of sand extraction under these consents, the Consent Holder must submit to the Council a Biosecurity Plan. As a minimum, the plan shall include a surveillance and response plan targeted at invasive marine species and unwanted organisms. The purpose of the Plan is to identify whether the transit of the dredge vessel between the Port of Auckland and the Mangawhai-Pakiri Embayment and the operation during the extraction might result in an avoidable spread of invasive marine species and unwanted organisms.

### Navigation

- On its northern approach to the Extraction Area, the extraction vessel shall not turn west toward the southern boundary of the Extraction Area until it has passed to the eastern side of the Navigation Waypoint shown in Appendix 4. The vessel shall pass to the east of this waypoint when leaving the Extraction Area.

MATAURANGA MAORI/EXPERT PANEL

- The Consent Holder must establish and maintain a Matautanga Maori/Expert Panel (Panel) for the purposes of expert oversight, assessment, and reporting for the life of

this consent, including input into any changes associated with monitoring or reporting cells. Within two weeks of the grant of consent, the Consent Holder will invite Omaha Marae, the Pakiri G and Taumata A & B blocks, Ngāti Manuhiri, Te Whānau o Pākiri (in addition to a representative of the Consent Holder) to collectively establish the Panel (who are not commissioned by the Consent Holder for the purposes of implementing this consent). This Panel will comprise an expert in each of the following areas:

  - Coastal processes

  - Marine ecology

  - Avifauna

  - Matauranga

  - Planning

Note: The individual discipline experts will represent the discipline, not a party.

- The purpose and function of the Panel shall include the following matters:

  - Review and provide feedback on the draft Management Plans prior to submission to the Council for certification,

  - Review the monitoring reports (including peer review comments provided by the SQEP(s) in relation to those monitoring reports) prepared in line with the Consent Conditions, including the sand extraction records, and advise the Consent Holder and the Council within ten working days of receiving the reports where the analysis undertaken by the Panel identifies sand extraction inconsistent with the SEMR,

  - Provide advice to the Consent Holder where the Consent Holder proposes to change or amend provisions of any certified Management Plans and provide the Council with a copy of this advice;

  - Make recommendations to the Consent Holder in respect of findings, and provide Council with a copy of those recommendations;

  - Make recommendations to the Consent Holder as to the form appropriate for the circulation of information to the following parties:

    - Ngāti Manuhiri

    - Te Whānau o Pākiri

    - Omaha Marae

    - Pakiri G Ahu Whenua Trust and Taumata A & B blocks

  - Prepare a Statement of Position in relation to the matters at a) – c) above for provision into the public domain within one month of each meeting.

- The Consent Holder will within 2 weeks of the establishment of the Panel invite the Panel to set the date of its first meeting.

- Following the first meeting, it is anticipated that the Panel will meet three monthly or at such greater intervals as it otherwise decides for the life of the consent.

- The Consent Holder shall coordinate the activities of the Panel including provision of secretarial services, timely provision of data and reporting, and coordination of Panel meetings.

- The Consent Holder shall meet the actual or reasonable costs, whichever at less, for the participation and contribution of the Panel members.

- The Consent Holder will supply the Panel with the documents listed below. In each case, the document will be provided in draft and in a timely fashion before being submitted to the Council by the Consent Holder to allow the Panel to consider the document and make recommendations to the Consent Holder should the Panel see fit:

  - Any Management Plans submitted to Council under Condition 24 (EMMP) & 32 (MMMP) including any EMMP submitted following a review required by Condition 31;

  - Any PSEAR submitted to Council under Condition 38

  - The SEMRs required to be submitted to Council under Condition 44.

  - The topographical survey results referred to in Condition 50.

  - Any application to change the sand extraction and/or discharge method or change of extraction vessel under Condition 21.

- The Consent Holder will supply the Panel with a copy of the following documents at the same time these are provided to the Council:

  - The outputs of the annual survey referred to in Condition 10 and the survey referred to in Condition 14 relating to the Extraction Exclusion Area.

  - Any report of any incident which results in injury or mortality to a marine mammal as referred to in Condition 34.

  - The records of sand extraction events referred to in Conditions 36.

Advice Note: The Council has the ability to consider the minutes of the Panel for the purposes of monitoring, and ensuring compliance with, the conditions of consent.

## COMMUNITY LIAISON GROUP

- The Consent Holder must within 2 months from the commencement of these consents, invite Friends of Pakiri Beach, Environs Holdings Limited (for the Te Uri o Hau Settlement Trust), Manuhiri Kaitiaki Charitable Trust, the Surfbreak Protection Society, Omaha Marae, Te Whanau o Pakiri, Department of Conservation, Mangawhai Harbour Restoration Society Inc, The Pakiri G Ahu Whenua Trust and Taumata A&B blocks to participate in a Community Liaison Group (“CLG”) comprising a representative of each of its members. The Consent Holder shall be a member of the CLG. The purpose of the CLG is to discuss matters related to the exercising of these consents, provide input into management and monitoring plans, and discuss monitoring results.

- Provided at least two of the parties listed in condition 66 confirm, in writing, that they wish to participate in the CLG, the consent holder must establish the CLG. Council may also participate as observers, should they wish to do so.

- Meetings of the CLG should be held annually in the vicinity of Pakiri/Leigh.

- The Consent Holder will be responsible for keeping and distributing meeting minutes and must meet the administrative costs of the CLG.

- The Consent Holder must submit a draft of the following documents to the CLG for written comment at least 20 working days prior to submitting it to Council for certification:

  - Any Management Plans submitted to Council under Condition 24 (EMMP) & 32 (MMMP) including any EMMP submitted following a review required by Condition 31;

  - Any PSEAR submitted to Council under Condition 38

  - The SEMRs required to be submitted to Council under Condition 44.

  - The topographical survey results referred to in Condition 50.

  - Any application to change the sand extraction and/or discharge method or change of extraction vessel under Condition 21.

- The Consent Holder must provide any feedback received from the CLG on these documents to Council at the time they are submitted for certification, along with any explanation of where any comment suggesting changes to the documents has not been incorporated and the reasons why.

- The Consent Holder will supply the CLG with a copy of the following documents at the same time these are provided to the Council:

  - The outputs of the annual survey referred to in Condition 10 and the survey referred to in Condition 14 relating to the Extraction Exclusion Area.

  - Any report of any incident which results in injury or mortality to a marine mammal as referred to in Condition 34.

  - The records of sand extraction events referred to in Conditions 36.

Advice Note: The Council has the ability to consider the minutes of the CLG for the purpose of monitoring, and ensuring compliance with, the conditions of consent.

### Advice notes

- Any reference to number of days within this decision refers to working days as defined in s2 of the RMA.

- For the purpose of compliance with the conditions of consent, “the council” refers to the council’s monitoring officer unless otherwise specified. Please email monitoring@aucklandcouncil.govt.nz to identify your allocated officer.

- For more information on the resource consent process with Auckland Council see the council’s website: www.aucklandcouncil.govt.nz. General information on resource consents, including making an application to vary or cancel consent conditions can be found on the Ministry for the Environment’s website: www.mfe.govt.nz.

- If you disagree with the monitoring charge specified in Condition 4 or any additional charges relating to the processing of the applicants(s), you have a right of objection pursuant to sections 357A and/or 357B of the Resource Management Act 1991. Any objection must be made in writing to the council within 15 working days of your receipt of this decision (for s357A) or receipt of the council invoice (for s357B).

- The consent holder is responsible for obtaining all other necessary consents, permits, and licences, including the Heritage New Zealand Pouhere Taonga Act 2014. This consent does not remove the need to comply with all other applicable Acts (including the Wildlife Act 1953, Marine Mammals Protection Regulations 1992 and the Health and Safety at Work Act 2015), regulations, relevant Bylaws, and rules of law.

APPENDIX ONE:	DOCUMENTATION REFERRED TO IN CONDITION 1 [TO BE UPDATED]

| Report title and reference | Author | Rev | Dated |
| --- | --- | --- | --- |
| Auckland Offshore Sand Extraction Site – Review of Coastal Processes Effects | Beca | D | 15 July 2019 |
| Assessment of Ecological Effects: Following Sand Extraction from Auckland Offshore Sand Extraction Site | Bioresearches | 1 | 24 July 2019 |
| The Economic Contribution and Impact of Pakiri Sand Extraction | M.E. Consulting | - | 7 August 2019 |
| Deep Sand Dredging from Hauraki Gulf, Mangawhai Heads to Te Arai Point: Heritage Assessment | Clough & Associates | - | 2011 |
| Assessment of Underwater Noise Effects Sand Extraction – Auckland Offshore Extraction Area Mangawhai – Pakiri Embayment | Styles Group | 2 | 31 March 2020 |
| Assessment of Airborne Noise Effects Sand Extraction – Auckland Offshore Extraction Area Mangawhai – Pakiri Embayment | Styles Group | 2 | 25 February 2020 |
| Pakiri Sands, Multibeam Echo-Sounder Survey December 2018 Report of Survey | Survey Worx | 3 | 23.05.2019 |
| Kaipara Ltd Offshore Sand Extraction: Marine Mammal Assessment of Effects | Cawthron |  | April 2020 |

| Surf Break Impact Assessment: Mangawhai-Pakiri Kaipara Excavators Ltd Offshore Dredging Consent | eCoast | 2 | 2 September 2020 |
| --- | --- | --- | --- |
| Draft Environmental Monitoring Management Plan (EMMP) | MBL | Draft for Reply | 22/02/2022 |

| Other additional information | Author | Rev | Dated |
| --- | --- | --- | --- |
| Further Information Response: RE: CST60343373-S92 Response and Public Notification Request Including: - Assessment of Underwater Noise Effects (Styles Group): - Assessment of Airborne Noise Effects (Styles Group): - S92 Response – Ecology (Bioresearches) - S92 Response – (Beca) - Surveyworkxs Report of Survey; and - Marine Mammal Assessment of Effects (Cawthron) | Collated by David Hay of Osborne Hay | - | April 14 2020 |
| Email: RE Kaipara Ltd Offshore Sand Extraction Site Application – Change to Site Boundary | David Hay of Osborne Hay | - | 1 July 2020 |

| Email: Kaipara Ltd – Additional Reasons for Resource Consent | David Hay of Osborne Hay | - | 1 July 2020 |
| --- | --- | --- | --- |
| Including: |  |  |  |
| - Letter: Kaipara Limited / CST60343373 / Discharges, prepared by Morgan Slyfield, dated 10 September 2020 |  |  |  |
| - Assessment: Re: Kaipara Ltd Application – Assessment of Discharges and Disposal, prepared by David Hay of Osborne Hay (North), dated 31 August 2020. |  |  |  |
| Cultural Effects Assessment | Te Uri o Hau |  | August 2020 |

APPENDIX TWO:	SENSITIVE BENTHIC COMMUNITIES

| Habitat | Primary indicators |
| --- | --- |
| Beds of large bivalve molluscs | A bed of large bivalves exists where living specimens of bivalve species: - are estimated to cover 30% or more of the seabed on average in visual images of either 1m2 or lateral view; or - comprise 30% or more by average weight or volume in grab samples. Large bivalves include: Horse mussels (Atrina zelandica) Scallops (Pecten novaezelandiae) Large dog cockle, (Tucetona laticostata) Dredge oysters (Ostrea chilensis) Green lipped mussels (Perna canaliculus) Geoducks (Panopea zelandica and P. smithae) Trough Shells (Spisula discors and S. murchisoni) Triangle Shell (Crassula aequilatera) Shellfish known to pass through dredge alive at greater than 90% are excluded; Clam (Dosinia anus, D. subrosea, Bassina yatei) Myadora sp. |
| Brachiopod beds | A brachiopod bed exists if: - one live brachiopod occurs per m² of seabed sampled using seabed photographs; or - one or more live specimens occur in grab samples. |
| Bryozoan thicket | A bryozoan thicket (here the term thicket is used synonymously with the terms bed, reef, meadow, etc.) is present if: - colonies of large frame-building bryozoan species cover at least 50% of the seabed in visual imaging surveys; - one or more colonies of large frame building bryozoan species occur per m² of seabed sampled using towed sampling gear; or - one or more large frame building bryozoan species is found in grab samples. |
| Calcareous tube worm thickets | A sensitive tube worm thicket is present if: - 2 or more colonies of a mound forming species of tube worm are found in any grab sample; or - 2 or more colonies are observed at a greater than 10% coverage in a visual image, either 1m2 or lateral view. |
| Chaetopteridae worm fields | A sensitive Chaetopteridae worm field is present if worm tubes and/or epifaunal species: - contribute 25% or more of the volume of a sample collected in a grab sample; or - colonies of tube worm species cover at least 50% of the seabed in visual imaging surveys. |
| Macro-algae beds | Detection of a single occurrence of any fixed specimen of a red, green or brown macroalga at greater than 30% cover is sufficient to indicate that this habitat has been encountered. |
| Rhodolith (maerl) beds | A rhodolith bed exists if: - a single specimen of a rhodolith species is found in grab sample; or - there is more than 10% cover of living coralline thalli in visual images. |

| Sea pen field | A sea pen field exists if: - one or more specimens of any species of sea pen is found in a grab sample; or - two or more specimens per m² are found in seabed imaging surveys. |
| --- | --- |
| Sponge gardens | A sponge garden exists if metazoans of Class Demospongiae, Class Hexactinellida, Class Calcarea or Class Homoscleromorpha: - are estimated to cover 25% or more of the seabed in visual images of either 1m2 or lateral view. |

APPENDIX THREE:	BEACH SURVEYING VOLUME COMPARTMENTS

APPENDIX Four: Proposed Offshore Sand Extraction Area showing Monitoring and Extraction Cells, and Control Areas with Monitoring Cells. Dated 6/7/2023.