Subdivision conditions

These conditions are draft.

The following notes guide the reader as follows:

- Yellow highlights require cross referencing that will be finalised later in the process.

- Blue highlights refer to lot numbers that will change with the subdivision scheme revision.

- Greyed text relates to the retirement village (being the conditions the Panel would have imposed if it had found that consent could have been granted for this part of the Project)

Resource Consent:		101.2025.13180

Grants to:				Matamata Development Limited

Commencement date:		Insert Decision Date

Lapse Date:			5 years after commencement date

Expiry date:			No expiry date

Location: 			Station Road, Matamata (Lot 1 Deposited Plan South Auckland 65481, Lot 2 Deposited Plan 567678, Lots 1 and 2 Deposited Plan 21055, Lots 4 and 5 Deposited Plan 384886, Lot 204 Deposited Plan 535395 and Lots 25 and 106 Deposited Plan 393306, Lot 3 Deposited Plan South Auckland 14362)

The activity: Subdivision consent (Section 11 of the Resource Management Act 1991) to subdivide land to develop approximately 95ha of land for residential purposes, a neighbourhood centre, a retirement village, two solar farms and ancillary infrastructure.

Advice note: Consent to cancel the consent notices, covenants and easements on the existing Records of Title will need to be obtained. The Fast Track Approvals Act 2024 does not provide for the cancellation of consent notices pursuant to Section 221(3), Section 241(3) and Section 243(e) of the Resource Management Act 1991.

This consent must be read in conjunction with:

- 102.2025.13180 - : Land use consent (Section 9 of the Resource Management Act 1991 (RMA) and under the National Environmental Standard for Assessing and Managing Contaminants in Soil to Protect Human Health 2011) to develop approximately 95ha of land for residential purposes, a neighbourhood centre, a retirement village, two solar farms and ancillary infrastructure.

And will be interpreted with guidance from the following glossary:

| (The) Council | Matamata-Piako District Council |
| --- | --- |
| JOAL | Jointly Owned Access Lot |
| RMA | Resource Management Act 1991 |
| Site | Collectively, Lot 1 Deposited Plan South Auckland 65481, Lot 2 Deposited Plan 567678, Lots 1 and 2 Deposited Plan 21055, Lots 4 and 5 Deposited Plan 384886, Lot 204 Deposited Plan 535395 and Lots 25 and 106 Deposited Plan 393306, Lot 3 Deposited Plan South Auckland 14362. |
| SQEP | Suitably Qualified and Experienced Person |

And is subject to the following conditions:

General conditions

- The subdivision will be carried out in general accordance with the application for resource consent, including any reports, plans, and further information (listed in Appendix [1]) provided by the Consent Holder, and in accordance with the following conditions of consent. Where there is any apparent conflict between the application and consent conditions, the consent conditions will prevail.

This resource consent must not be exercised until:

- The Consent Holder has supplied an electronic file/(s) containing all documents referred to in Appendix 1, indexed and referenced to enable easy access, to the satisfaction of the Council as to compliance with the following standards:

- Each document must be allocated a unioque document number.

- The file containing the document must be named in accordance with the following convention:

Unique Document Number-Document Title-Author-Document date-Document Version

Documents that have been superseded must be marked ”superseded”, with reference to the final version; and:

- A Private Developer Agreement (PDA) in a form and on terms satisfactory to Council in all respects has been entered into with the Council and approved by Waikato Waters Limited, which includes provision for all of the following:

- Agreement on the land to vest in Council as part of the development including, but not limited to, agreement as to the size, purpose and value of such land;

- Agreement on the quantum of development and financial contributions payable to Council less any offset on account of the value of land to vest in Council;

- The quantum and timing of payments to Council; and:

- Provision of securities as required by Council in its sole discretion securing the performance of the Consent Holder’s obligations under the PDA, which may include a statutory land charge, caveat or encumbrance on land forming part of the development.

The PDA shall be duly executed and all required securities registered in accordance with the PDA.

- For the purposes of this consent, any reference to ‘Site’ means land legally described as Lot 1 Deposited Plan South Auckland 65481, Lot 2 Deposited Plan 567678, Lots 1 and 2 Deposited Plan 21055, Lots 4 and 5 Deposited Plan 384886, Lot 204 Deposited Plan 535395 and Lots 25 and 106 Deposited Plan 393306, Lot 3 Deposited Plan South Auckland 14362 prior to any further subdivision of the land.

- The Consent Holder must retain the services of a suitably qualified and experienced person (SQEP) to oversee the design and construction of any infrastructure required for the development. This person shall be responsible for ensuring adherence to approved construction plans, quality systems, and project completion requirements, acting as the Consent Holder’s representative. Prior to any design and development, the Consent Holder must advise the Matamata-Piako District Council (the Council) of the name and contact details of the representative. All discussions and correspondence relating to design and development matters shall be undertaken with Council staff only through this representative.

Advice note: All correspondence with Council required by these conditions of consent should be sent via email to Resource Consents@mpdc.govt.nz with reference to consent number 101.2025.13180.

- The Consent Holder must pay to the Council any administrative charge fixed in accordance with Section 36 of the RMA, or any charge prescribed in accordance with regulations made under Section 360 of the RMA.

Advice notes:

    - This includes the reasonable costs incurred by Council arising from supervision and monitoring of this consent, e.g. routine inspection of the Site by Council officers or agents, liaison with the Consent Holder, responding to complaints or enquiries relating to the Site, and review and assessment of compliance with the conditions of consents.

    - that pursuant to Section 332 of the RMA, enforcement officers may at all reasonable times go onto the property that is the subject of this consent, for the purpose of carrying out inspections, surveys, investigations, tests, measurements or taking samples.

    - The Section 224(c) RMA Certificate will not be issued until all fees relating to that development / subdivision stage are paid.

Staging

- Subdivision will be undertaken in stages, subject to each stage:

- Occurring sequentially. This does not preclude stages being subdivided concurrently.

Advice note: Sequentially means Stage 1, then Stage 2, then Stage 3 etc. Meaning that if subdivision in Stage 8 is to occur all numerically preceding stages must be completed or commenced.

- Complying with all relevant conditions in this resource consent.

- Being in general accordance with the plans, drawings, and information listed in Appendix [1].

- Able to be serviced in accordance with the conditions of this consent and does not restrict future development of stages.

- Except that, the subdivision of the retirement village may only occur after Stage 7 of the residential development is subdivided.

- Stages 5 and 6 of the residential development will be reconfigured into a single stage (hereafter ‘Amended Stage 5’) and all development shown on Highly Productive land (LUC-1 or -2 as defined by the New Zealand Land Resource Inventory) will be deleted.

- The Consent Holder will increase the size of all lots adjoining a Rural or Rural Residential Zone (outside the Site) to a minimum average of 1,500m2 (net), with a minimum lot size of 1,200m2 (net).

- Where variations to stages are proposed, or required by these conditions of consent, the Consent Holder will submit amended staging plans to the Council for review and approval.

- The residential, commercial and greenway staging is as follows:

- Day 0 subdivision as shown on Maven plan “Proposed Scheme Plan” (Ref: SHEETS 1 – 3, Rev M, dated February 2026) and amended as necessary by Condition [12].

- Stage 1A: Residential Lots 1 – 15, 52, 53, 58 – 61 and 65 – 68 (including Jointly Owned Access Lot (JOAL) 3016); Lots 4001 and 4002 to be vested as Local Purpose – Stormwater Reserve (Stormwater Basin A); and Road 1 (no lot number assigned on scheme plan).

Advice note: as shown on the following Maven plan “Proposed Land Use Consent Stage 1A” (Ref: C160-1A, Rev C, dated June 2025)

- Stage 1B: Residential Lots 16 – 30 and 54 – 56; and Lot 3032 to be vested as Road (Roads 8 and 13).

Advice note: as shown on the following Maven plan “Proposed Land Use Consent Stage 1B” (Ref: C160-1B, Rev C, dated June 2025)

- Stage 1C: Residential Lots 37 – 45, 47 – 51, 57 and 62 – 64; and Lot 3033 to be vested as Road (Roads 13 and 16).

Advice note: as shown on the following Maven plan “Proposed Land Use Consent Stage 1C” (Ref: C160-1C, Rev D, dated November 2025)

- Stage 2A: Residential Lots 69 – 73, 77 – 82, 107 – 112 and 123 – 132 (including JOAL 3017 and 3018); and Lot 3002 to be vested as Road (Roads 1 and 9).

Advice note: as shown on the following Maven plan “Proposed Land Use Consent Stage 2A” (Ref: C160-2A, Rev C, dated June 2025)

- Stage 2B: Residential Lots 104 – 106, 113 – 122 and 133 – 145; and Lot 3003 to be vested as Road (Roads 1, 9 and 12).

Advice note: as shown on the following Maven plan “Proposed Land Use Consent Stage 2B” (Ref: C160-2B, Rev C, dated June 2025)

- Stage 2C: Residential Lots 74 – 76 and 83 – 103; and Lot 3034 to be vested as Road (Roads 14, 15 and 16).

Advice note: as shown on the following Maven plan “Proposed Land Use Consent Stage 2C” (Ref: C160-2C, Rev C, dated June 2025)

- Stage 3: Residential Lots 146–217 (including JOAL 39 and 3020); Lot 4003 to be vested as Local Purpose – Stormwater Reserve (Stormwater Basin B and the greenway); Lot 5001 to be vested as Local Purpose - Wastewater Reserve (central wastewater pump station); Lot 6001 to be vested as xxx and Lots 3004 and 3005 to be vested as Road (Roads 10, 14 and 16).

Advice note: as shown on the following Maven plan “Proposed Land Use Consent Stage 3” (Ref: C160-3, Rev C, dated June 2025)

- Stage 4: Residential Lots 218–277; Lot 1001 to be vested as Local Purpose – Recreation Reserve (open space at the commercial node); Lot 1002 (the commercial node); and Lots 3006 and 3007 to be vested as Road (Roads 1, 9 10 and 11).

Advice note: as shown on the following Maven plan “Proposed Land Use Consent Stage 4” (Ref: C160-4, Rev D, dated November 2025)

- Amended Stage 5: Residential Lots 278–316, 320–337, 338–357, 379–389 (including JOAL 3021 and 3022); and Lots 3008, 3009, 3010 and 3011 to be vested as Road (Roads 1, 4, 5, 6, 7 and 9).

Advice note: As shown on Maven plans (excluding the development on HPL land):

- “Proposed Land Use Consent Stage 5” (Ref: C160-5, Rev C, dated June 2025)

- “Proposed Land Use Consent Stage 6” (Ref: C160-6, Rev D, dated November 2025)

- Stage 6:

- Stage 7: Residential Lots 390 – 455 (including JOAL 3023); Lot 4004 to be vested as Local Purpose – Stormwater Reserve (Stormwater Basin C); and Lots 3012 and 3013 to be vested as Road (Roads 1, 2 and 4).

Advice note: As shown on the following Maven plan “Proposed Land Use Consent Stage 7” (Ref: C160-7, Rev C, dated June 2025)

- Stage 8A: Residential Lots 426 – 428, 456 – 485 and 495, 496, 503, 504 and 511 (including JOAL 3024 and 3025); and Lot 3014 (Roads 2 and 3).

Advice note: as shown on the following Maven plan “Proposed Land Use Consent Stage 8A” (Ref: C160-1A, Rev D, dated November 2025)

- Stage 8B: Residential Lots 486 – 494, 497 – 502, 505 – 510 and 512 – 518 (including JOAL 3026, 3027 and 3028); Lot 4005 to be vested as Local Purpose – Stormwater Reserve (Stormwater Basin D); Lot 5002 to be vested as Local Purpose – Wastewater Reserve (northern wastewater pump station); and Lot 3015 (Roads 1 and 2).

Advice note: as shown on the following Maven plan “Proposed Land Use Consent Stage 8B” (Ref: C160-1B, Rev D, dated November 2025)

- The subdivision of the retirement village activity (to establish three fee simple lots) will take place following construction of the retirement village, but no sooner than Stage 7 of the residential development is subdivided.

Advice note: As shown on the following Maven plan “Lots 1-3 Subdivision Scheme Plan” (Ref: C1000 SCHEME, Rev A, dated June 2025)

Day 0 subdivision

- Prior to the certification of the survey plan pursuant to Section 223 of the RMA and the issue of a certificate pursuant to Section 224(c) of the RMA the following requirements are to have been satisfied for the Day 0 subdivision:

- The Consent Holder will submit a Day 0 survey plan which will reflect the Day 0 Scheme Plan (referenced in Condition [10(a)]) submitted with the resource consent application and including the following adjustments:

- Combine Lots 1 and 2, being all ‘Balance Lot’ (and hold in the same title as Lot 3).

- Remove rural-residential Lots 8 and 9 (land to be incorporated into Lot 7).

Advice note: Lot numbers referred to in this condition refer to those shown on Maven plan “Proposed Scheme Plan” (Ref: SHEETS 1 – 3, Rev E, dated July 2025).

- Provide confirmation from a SQEP that:

- All water, power, and telecommunications supplies that cross the boundaries of Lots 1 – 6 hereon have been severed or are protected by easements.

- The wastewater disposal fields serving the dwellings on Lots 2 and 5 hereon are located entirely within the boundaries of the said lots.

- Any buildings or structures on Lots 1 – 6 hereon meet the minimum setback distances from the boundaries in compliance with the development standards for the underlying zone as set out in the Operative Matamata-Piako District Plan.

Construction requirements

- The carriageway for right of ways “C” and “D” are to be formed and constructed to an all-weather surface (at least a 100mm GAP 40 base-course or equivalent and a 20mm dressed layer of AP20) of at least 3m wide to the satisfaction of the Council. Adequate provision must be made for the disposal of stormwater runoff.

Easements

- The Day 0 survey plan will reflect the Day 0 easement schedule submitted with the resource consent application and the following adjustments:

- Remove Easements E and F.

- Include a suitable easement over the Balance Lot to protect the stormwater infrastructure required for Catchment 2, to the satisfaction of the Council.

- Right of Way marked “C” and “D” as shown on the Scheme Plan must have a legal width of no less than 6m.

- Right of Way marked “K” as shown on the Scheme Plan must have a legal width of no less than 3m and can be unformed. The easement must follow the boundary of the existing farm track and will be determined at the time of land survey, to the satisfaction of the Council.

- That the schedule of easements as described and submitted in accordance with the Day 0 survey plan be duly granted or reserved.

- Pursuant to Section 243(e) of the RMA, the conditions as to the creation of the easements marked:

- “X” on DP XXXX and created by Easement Instrument 7422421.6;

- “X” on DP XXXX and created by Easement Instrument 7390609.4;

- “X” on DP XXXX and created by Easement Instrument 7835210.3; and

- “X” on DP XXXX and created by Easement Instrument 12244095.3,

are to be cancelled.

- Pursuant to Section 243(e) of the RMA, the conditions as to the creation of the easements marked:

- “X” on DP XXXX and created by Easement Instrument 7422421.5, to the extent that xxxx; and

- “X” on DP XXXX and created by Easement Instrument 7422421.7, to the extent that xxxx,

are to be partially cancelled.

Amalgamation conditions

- The Day 0 survey plan will show the amalgamation conditions shown on the Day 0 Scheme Plan submitted with the resource consent application (see LINZ Ref: xxx), specifically:

- Lots 4 and 6 hereon will be held in the same Record of Title.

- Lots 1 (and 2) and 3 hereon will be held in the same Record of Title.

- That Lots 25 and 106 DP 393306 be transferred to the owners of Lot 100 DP 380025 and one Record of Title be issued to include all parcels.

- That Lots 3, 15 and 16 hereon be held in the same Record of Title.

- Pursuant to Section 241(3) of the RMA, the Council resolves to cancel, the amalgamation conditions requiring Lot 2 DP 567678 and Lot 2 DP 21055 to be held in the same Record of Title.

- Pursuant to Section 241(3) of the RMA, the Council resolves to cancel, the amalgamation conditions requiring Lot 204 DP 535395 and Lots 25 and 106 DP 393306 to be held in the same Record of Title.

Esplanade Reserve/ Streambed

- That Local Purpose (Esplanade) Reserve, shown on the scheme plan (Rev M, February 2026) as Lots 10, 11, 12, 13, 14, 21 and 22, be vested in the Council.

- That Lots 17, 18, 19, and 20 shown on the scheme plan (Rev M, February 2026) shall be vested in Council on deposit as Streambed

- No compensation will be paid by the Council to the Consent Holder for the esplanade reserve or streambed (to be vested in accordance with Conditions [12(k) and (l)]).

Consent Notice

- The following must be registered as a Consent Notice on the Record of Title to be issued for Lot 2 (and Lot 1):

“No future urban land use or development occur, other than those activities permitted by the MPDC District Plan for the zone within which the said lots are located or authorised prior to the date of this consent notice being registered.”

The following (as relevant) must be registered as a Consent Notice on the records of title to be issued for any of Lots 1, 3, 4, 5, and 6 and 7 that do not have operational domestic or farming water supplies or power or telecommunications connections or complying vehicle entrances:

At the time of subdivision the said lots did not have operational domestic water supplies/ farming water supplies/ power connections/ telecommunications connections/ vehicle entrances. Supplies/ connections/ complying vehicle entrances, when required, must be installed entirely at the cost in all matters of the then owner.

The following must be registered as a consent notice on the records of title to be issued for Lots 3, 4, 5, 6 and 7:

“No future  land use or development occur, other than those activities permitted by the MPDC District Plan or authorised by the Ashbourne Landuse and Subdivision consents (MPDC ref: 2025.13180). The said lots are intended for future development in accordance with the said land-use and subdivision consents and do not have connections to reticulated urban services. Servicing of the future urban development on Lots 4 - 6 hereon will be established in accordance with the conditions of the said Ashbourne consents and the Private Developer Agreement between the Consent Holder and MPDC dated xxx. The proposed solar farms on Lots 3 and 7 hereon do not have connections to urban services or complying vehicle entrances. Services to supply the solar farms and vehicle entrances must be installed at the time of development in accordance with the said landuse consents and entirely at a cost in all matters of the Consent Holder.

- Pursuant to Section 221(3) of the RMA, the Council resolves to cancel the following Consent Notices:

- Consent Notice 7422421.2 on Lot X DP XXX held in Record of Title xxx;

- Consent Notice 7422421.3 on Lot X DP XXX held in Record of Title xxx;

- Consent Notice 7390609.2 on Lot X DP XXX held in Record of Title xxx; and

- Consent Notice 12244095.2 on Lot X DP XXX held in Record of Title xxx.

Section 223 Requirements – All Stages (excluding Stage 0)

Prior to requesting approval under Section 223 of the Resource Management Act 1991, the consent holder must provide to Council a written statement from a Licensed Cadastral Surveyor, stating:

- whether all services to a lot (including any pipes, cables, pumps and ancillary equipment) are located entirely within the lot to which those services relate; or

- where a service to a lot requires rights over an adjacent lot or other land in order to transport or connect the service to that lot, the Land Transfer Plan shows the nature and location of any easement to be created for those services, including that due allowance has been made to ensure services are constructed within the relevant easement area and that services do not interfere with the construction of any dwelling or other improvement located on the servient land.

The statement provided by the Licensed Cadastral Surveyor in accordance with this condition shall be accompanied by all evidence reasonably required by the Council to verify the nature and location of the services and the rights of each lot to connect services through other land.

- The Consent Holder must submit a survey plan for certification by the Council. The survey plan must be in general accordance with the subdivision scheme plans referenced in Condition [10] for each stage or sub-stage and show all lots to vest in the Council (including roads and reserves) and all easements and amalgamation conditions required by this consent.

- The survey plan must be accompanied by:

- The engineering plans approved as part of MPDC Landuse Consent 102.2025.13180 showing (but not limited to):

- The provision of reticulated 3-water infrastructure.

- The provision of individual water, wastewater, power, fibre and telecommunications connections onto Lots to be created by the Stage.

- The provision of stormwater devices and connections for all roadways and public areas.

- The location and design of vehicle crossings onto Lots to be created by the Stage.

- The design and dimensions, formation, and construction details for all public roads, including car parking, loading bays, manoeuvring areas, pedestrian and cycle paths, and road markings and signage to be included in the Stage.

- The provision of overland flow paths.

- Identification of any additional easements for infrastructure not shown on the Scheme Plans.

- The landscaping plans for all public roads, public park/s and stormwater reserves to be vested in the Council approved as part of MPDC Landuse Consent 102.2025.13180

- A road naming application (in accordance with the Council’s Numbering of Properties, Naming of Roads, Access Ways and Open Spaces Policy).

## Land to Vest in Council

- Where a stage or substage includes land to be vested, the survey plan will show:

- In respect to Lots 4001, 4002, 4003, 4004, 4005, 4006 (on the approved resource consent subdivision plans referenced in Appendix [1]), the lots must be vested in the Council as Local Purpose (Stormwater) Reserve. The Consent Holder must meet all costs associated with the vesting of the Local Purpose (Stormwater) Reserve/s.

Advice note: Local Purpose (Stormwater) Reserves shall be vested in Council free of charge.

- In respect of Lot 1001 (on the approved resource consent subdivision plans referenced in Appendix [1]) the lot must be vested in the Council as Local Purpose (Recreation) Reserve.  The Consent Holder must meet all costs associated with the vesting of the Local Purpose (Recreation) Reserve.

Advice Note: The purchase of the recreation reserve is addressed in the PDA.

- In respect of Lots 3001 – 3015 and Lots 3032 – 3034 (on the approved resource consent subdivision plans referenced in Appendix [1]) the lots must be vested in the Council as Local Purpose (Road) Reserve. The Consent Holder must meet all costs associated with the vesting of the Local Purpose (Road) Reserve/s.

- In respect of Lots 5001 – 5002 (on the approved resource consent subdivision plans referenced in Appendix [1]), the lots must be vested in the Council as Local Purpose (Wastewater) Reserve. The Consent Holder must meet all costs associated with the vesting of the Local Purpose (Wastewater) Reserve/s.

Advice note: Local Purpose (Wastewater) reserves shall be vested in Council free of charge.

## Amalgamation Conditions (Jointly Owned Access Lots)

- Where a stage or substage includes a JOAL, the survey plan will show:

- That Lot 3016 hereon (legal access) must be held in six undivided one sixth shares by the owners of Lots 59 – 61 and Lots 65 – 67 hereon as tenants in common in the said shares and individual Records of Title be issued in accordance therewith.

- That Lot 3017 hereon (legal access) must be held in eight undivided one eighth shares by the owners of Lots 70 – 73 and Lots 77 – 80 hereon as tenants in common in the said shares and individual Records of Title be issued in accordance therewith.

- That Lot 3018 hereon (legal access) must be held in six undivided one sixth shares by the owners of Lots 125 – 130 hereon as tenants in common in the said shares and individual Records of Title be issued in accordance therewith.

- That Lot 3019 hereon (legal access) must be held in two undivided one-half shares by the owners of Lots 159 and 160 hereon as tenants in common in the said shares and individual Records of Title be issued in accordance therewith.

- That Lot 3020 hereon (legal access) must be held in three undivided one third shares by the owners of Lots 179 – 181 hereon as tenants in common in the said shares and individual Records of Title be issued in accordance therewith.

- That Lot 3021 hereon (legal access) must be held in six undivided one sixth shares by the owners of Lots 309 – 311 and Lots 314 – 316 hereon as tenants in common in the said shares and individual Records of Title be issued in accordance therewith.

- That Lot 3022 hereon (legal access) must be held in four undivided one fourth shares by the owners of Lots 344, 345, 348 and 349 hereon as tenants in common in the said shares and individual Records of Title be issued in accordance therewith.

- That Lot 3023 hereon (legal access) must be held in two undivided one-half shares by the owners of Lots 419 and 420 hereon as tenants in common in the said shares and individual Records of Title be issued in accordance therewith.

- That Lot 3024 hereon (legal access) must be held in three undivided one third shares by the owners of Lots 427 – 429 hereon as tenants in common in the said shares and individual Records of Title be issued in accordance therewith.

- That Lot 3025 hereon (legal access) shall be held in five undivided one fifth shares by the owners of Lots 463 – 467 hereon as tenants in common in the said shares and individual Records of Title be issued in accordance therewith.

- That Lot 3026 hereon (legal access) must be held in two undivided one-half shares by the owners of Lots 488 and 489 hereon as tenants in common in the said shares and individual Records of Title be issued in accordance therewith.

- That Lot 3027 hereon (legal access) must be held in three undivided one third shares by the owners of Lots 494, 497 and 498 hereon as tenants in common in the said shares and individual Records of Title be issued in accordance therewith.

- That Lot 3028 hereon (legal access) must be held in four undivided one forth shares by the owners of Lots 501, 502, 505 and 506 hereon as tenants in common in the said shares and individual Records of Title be issued in accordance therewith.

Easement conditions

- Where a stage or substage includes an easement, the survey plan must show the easement along with appropriate easement details in a schedule.

- The surveying and definition of the easements and the preparation and registration of the easement documents must be completed by, and at a cost of, the Consent Holder. The easements must be:

- At least 1.5m either side of any wastewater or stormwater pipe invert where the pipe is less than 4m deep.

- At least 3m either side of any wastewater or stormwater pipe invert where the pipe is more than 4m deep.

The easement documents in favour of the Council shall be prepared by Council’s Legal Officer or Solicitor, and the surveying and definition of the easements and the preparation and registration of the easement documents shall be completed by, and at a cost in all matters to the Consent Holder.

- That the schedule of easements as described and submitted in accordance with each survey plan be duly granted or reserved.

Section 224(c) Requirements – All Stages (excluding Stage 0)

- Prior to the issue of the Section 224(c) certificate for any Stage (other than Stage 0), the Consent Holder must:

- Provide confirmation from a SQEP that all lots have legal and physical access to a road, connections to all reticulated network utilities for water, wastewater, stormwater, power, fibre and telecommunications, and overland stormwater flow paths to a Council-approved system, including:

- That the JOAL have been satisfactorily designed and constructed in general accordance with the approved engineering plans approved by MPDC Landuse Consent 102.2025.13180.

- That the new vehicle crossing/s to serve all JOAL have been designed and formed in general accordance with the engineering plans approved by MPDC Landuse Consent 102.2025.13180, including an at-grade (level) pedestrian footpath across the length of the crossing, using the same materials, kerbing, paving, patterns and finish as the footpath on each side of the crossing.

- Comply with all the conditions of consent relevant to the stage, including the conditions of MPDC Landuse Consent 102.2025.13180..

- Provide confirmation from the Council that all necessary works to ensure there is available servicing capacity to facilitate development have been completed to the extent required for that stage.

- Provide confirmation from the utility providers that works have been satisfactorily undertaken.

- Provide and install road naming signs in accordance with the Council standards for both public and private roads that serve six or more lots within the subdivision. The names must be as approved by the Council.

- Implement all hard and soft landscape works within the stormwater reserves, public parks, public roads and accessways in general accordance with the landscape plans approved by MPDC Landuse Consent 102.2025.13180.

- Provide the certified Landscape Maintenance Plan for all planting and landscaping to be established in all stormwater reserves, public parks and public roads approved as part of MPDC Landuse Consent 102.2025.13180.

- Provide as-built plans of completed landscape works (hard and soft) within all stormwater reserves, public parks and public roads in CAD (NZTM 2000) and pdf form in general accordance with the Development Engineering as-built requirements v1.3. Plans must be provided to the Council and include the following details:

- Asset names.

- All finished hard and soft landscape asset locations and type and any planted areas must be shown to scale with the square metres of planting annotated.

- All underground services and drainage.

- All paint colours, pavers, and concrete types with names of products to be included on the assets schedule; and

- An uncompleted works bond will be entered into where any landscape works required by the certified landscape plans have not been completed in general accordance with the certified plans. This may apply to matters such as street tree planting and riparian planting so that planting can be implemented at the most appropriate planting season. The bond amount shall be 1.5 x the contracted rate of any outstanding works and shall be agreed in consultation with the Council prior to lodging the bond. The liability of the Consent Holder must not be limited to the amount of the bond.

- Provide confirmation that the buffer planting required by MPDC Landuse Consent 102.2025.13180 has been implemented.

Advice note: See also Condition [21(a)], which requires a Consent Notice in respect of the buffer planting.

- Provide the Council a refundable bond in respect of the maintenance of the landscaping works required by the conditions of this consent. The maintenance bond will be held for a period of two years from the issue of the certificate under Section 224(c) for all stormwater reserves, public parks and public roads. The amount of the bond will be 1.5 x the contracted rate for two years’ maintenance.

- Provide a Geotechnical Completion Report from a SQEP confirming that all lots are stable and suitable for development.

Consent Notices

That all Consent Notices to be cancelled or required to be registered under the conditions of this resource consent, shall be prepared by Council’s Legal Officer or Solicitor and the preparation and registration of the Consent Notices shall be completed by, and at a cost in all matters to the Consent Holder.

Advice Note: Where consent notices refer to design plans or technical information, Council’s document number should be included in the wording of the consent notice, for ease of reference.

- The Consent Holder will provide wording to the Council for approval to address the following matters to be contained in Consent Notices when seeking Section 224(c) certificate for a Stage:

- A Consent Notice relating to the buffer planting on Lots 378-403, generally worded as follows:

“That the 4m strip of buffer planting planted along the rear or side boundary of the property (whichever is relevant) by the Developer will be maintained in perpetuity. Any dead or unhealthy plants within the buffer area are to be replaced during the next available planting season to maintain the visual buffer.”

- A Consent Notice relating to a building line restriction on Lots 38-45, 47-97, 166-180, 378-388, 390-403, 420, 428, 429, and 474-488, generally worded as follows:

“That a 8m building line restriction be imposed along the rear and/or side boundaries to restrict the construction, placement, or establishment of any Building. ‘Building’ will have the same meaning as the Building Act 2004.”

- A Consent Notice on Lots 38-45, 47-97, 166-180, 378-388, 390-403, 420, 428, 429, and 474-488, relating to the adjoining organic farming operation and generally worded as follows:

“That that no sprays or treatments may be used on the property that do not comply with organic farming practices.”

- A ‘no complaints’ Consent Notice on Lots 38-45, 47-97, 166-180, 378-388, 390-403, 420, 428, 429, and 474-488 generally worded as follows:

“That the lot owner shall make no complaint, submission, appeal, or objection in relation to the lawful farming practices on adjoining land.”

- A Consent Notice on Lots 38-45, 47-97, 166-180, 378-388, 390-403, 420, 428, 429, and 474-488 generally worded as follows

“That dwellings are limited to a maximum height of 6m / one storey.”

- A Consent Notice relating to a landscaping strip on Lots 108 and 109, generally worded as follows:

“That the 2m strip of landscape planting along the eastern boundary planted by the Developer will be maintained in perpetuity. Any dead or unhealthy plants within the buffer area are to be replaced during the next available planting season to maintain the visual buffer.”

- A Consent Notice on Lots 107 – 110 delaying development on the lots, generally worded as follows:

“That the construction of dwellings on Lots 107 – 110 must occur no earlier than as part of Stage 3 of the Ashbourne development.”

h.   A consent notice on the RTs of all lots that require onsite stormwater management devices as determined by the SMP, generally worded as follows:

That the owners/occupiers shall, at the time of building construction, install the onsite stormwater management devices in accordance with the requirements of the SMP (see MPDC Landuse Consent 102.2025.13180) and thereafter maintain the onsite stormwater management devices, in perpetuity, in accordance with the On-lot Devices Management Plan which forms part of WRCXXXXX .

i. A Consent Notice on all developable lots to require that future development be in accordance with MPDC Landuse Consent 102.2025.13180.

Advice note: Any development not in accordance with the said landuse consent shall be subject to approval of a s127 RMA variation of the conditions of the said landuse consent, or a separate landuse consent applied for under s88 RMA.

j. A consent notice on the RTs of all Balance Lot/s created at each stage, to require that the owner/s of the said Balance Lot/s

- Shall be responsible for the maintenance of the Stormwater Management Devices vested in Council to serve Stage XX;

- Maintenance shall be in accordance with the SMP (see MPDC Landuse Consent 102.2025.13180); and:

- Maintenance shall be undertaken for a period of 24 months from [Date] {insert the date of issuing the Section 224 certificate under the Resource Management Act 1991 for the relevant subdivision stage/ sub-stage that requires the devices to be vested in Council.

k. Consent notice to require ongoing compliance with EcoRMP?

l. A consent notice on the RTs of all developable lots adjoining Local Purpose Reserves (stormwater/ wastewater and recreation) to be vested in Council to record that permeable fences that comply with the standards in Landuse Consent 102.2025.13180 must be erected at the time of building construction and thereafter maintained in perpetuity in compliance with the said standards.

m. A consent notice on the RTs of all developable lots to require that at the time of building two onsite carparks that comply with the standards in Landuse Consent 102.2025.13180 must be installed and thereafter maintained in perpetuity in compliance with the said standards.

n. A consent notice on the RTs of all developable lots that adjoin Road 1, and/or a shared path to require that provision be made at the time of building construction, and thereafter maintained in perpetuity, for vehicles to manoeuvre on-site in order to exit the lot in a forward direction.

o. A consent notice on the RTs of all developable lots that do not have constructed vehicle entrances at the time of application for s224 RMA certification, to require that vehicle entrances complying with RITS be installed at the time of building construction, and thereafter maintained in perpetuity.

Fencing covenant

That a fencing covenant as defined in Section 2 of the Fencing Act 1978 that protects the Council from liability to contribute towards any work and/or cost on the fence along the common boundary be registered on the Record of Tile of all lots adjoining Local Purpose Reserves (stormwater/ wastewater and recreation) to be vested in Council.

- All of the Consent Notices required by Condition [21] will be registered on the relevant Records of Title.

Land covenant conditions

- Where a stage or substage includes a JOAL, the Consent Holder must provide a copy of the draft land covenant document to the Council with the survey plan. The draft land covenant document must include provision for the following items:

  - Specifies ownership, and operation and maintenance requirements, of the private stormwater systems for the JOAL.

- Specifies responsibilities together with an acceptable method of management of the stormwater systems within the JOAL, and for the raising of funds from shareholders or members from time to time to adequately finance future maintenance and renewal obligations of the stormwater system.

- Specifies the operation and maintenance requirements of lot owners in respect of the private on-lot stormwater devices

Advice note: This will replicate, to the extent necessary, the On-lot Devices Management Plan which forms part of WRCXXXXX. If necessary, it will append this Management Plan in full to the Records of Title.

- The Consent Holder will supply a solicitor’s undertaking that the land covenants above, as approved by Council, will be registered with LINZ.

Development/ financial contributions

That the following contributions applicable to each stage be paid to Council in full, in cleared and available funds, prior to the issue of the subdivision completion certificate under section 224 of the Resource Management Act 1991 for the relevant subdivision stage:

- Financial contributions pursuant to Section 108 of the RMA and the Operative Matamata-Piako District Plan (District Plan);

- Development  contributions in accordance with the Development Contributions Policy that applies at the time of application for the subdivision completion certificate under section 224 of the Resource Management Act 1991 for the relevant subdivision stage; and:

- Contributions in accordance with the PDA.

Other completion conditions

The Consent Holder must comply with the “Other Completion Conditions” (Conditions 99 – 107) of Landuse Consent 102.2025.13180 for each stage of subdivision, prior to the issue of the subdivision completion certificate under section 224 of the Resource Management Act 1991 for the relevant subdivision stage.

APPENDIX 1

| Document | Author | Date | Document Version |
| --- | --- | --- | --- |